M screws standard:

The Clearance Hole Rule:

- Clearance Hole = Screw Diameter + 0.3 mm to 0.8 mm

- Why: Ensures smooth pass-through on 3D printers (which naturally contract slightly when printing inner diameters) while leaving maximum landing area for the screw head.

The Heat-Set Insert Hole Rule:

- Insert CAD Hole = Insert Outer Body Knurl Diameter minus 0.5 mm to 0.7 mm

- Why: This precise interference fit gives the melting plastic enough material to flow into the knurl grooves without bulging outward or squishing inward into the thread area.

The Insert Chamber Depth:

- Always model the insert hole 1.0 mm to 1.5 mm deeper than the physical length of the brass insert. This prevents molten plastic squeezed ahead of the tip from clogging the bottom threads.

The Entry Lead-in:

- Always add a 45∘ chamfer at the mouth of the insert hole (matching the insert's outer diameter). It cradles the insert so it stays vertical before you apply the soldering iron.

- Thread inserts M2: Outer diameter = 3.2mm, Height = 4mm

- Thread inserts M3: Outer diameter = 4.6 mm, Height 5.7mm

- Thread inserts M4: Outer diameter = 6.3mm, Height = 8.1 MM

- Thread inserts M5: Outer diameter = 7mm, Height = 9.5mm

- Threat inserts we do 0.1 mm less for a snug fit for it to melt into.